Lab: System and Mobility

Project introduction

1) Project name: Autonomous Stair Identification, Climbing, and Descending for Tracked Robots

2) Overview in 100 words:

Figure 1. The system setup

This project developed a controller for autonomous stair identification, climbing, descending for a tracked robot. First, the stair features is extracted from the perception system, then fuzzy logic controller is used to smoothly steer the robot on stair. The advantage of the proposed method is that it offers robust performance without the need for an accurate attitude estimator or detailed physical properties of the robot. Experimental results have verified that the controller is reliable for stair climbing and descending task in urban environments. Hence, the autonomous controller could be easily implemented on any tracked vehicles that are capable to be manually driven in a stair.

3) Infosheet: in preparation

4) Videos clips in urls/ flash/mp4. Title 1 URL1, Title 2 URL2, …

5) Invention years: 2013-2015

6) Key research high light:

- Innovative approach is used to allow the controller to be applied on both stair climbing and stair descending task. The controller uses multiple sensors to approach and track the stair. A head platform driven by a servo is used to direct an IP camera and a laser scanner to the stair. An inertial measurement unit (IMU) is used to give estimate of the robot pose. A combination of the stair Identification through Hough transform from RGB image, and stair feature tracking from tilting 2D laser scan is used to approach and align to the stair.

- Fuzzy logic controller with two inputs and two outputs is used to smoothly steer the robot on the stair. The inputs are the offset error and heading errors from laser scan data, and the outputs are velocity and turn of the robot. The fuzzy rules are designed to maintain perpendicular heading to the stair while minimizing slippage and corrective turns overshoot on the stair.

- By 2015, the method has been adapted successfully on at least three different track robots with varying size and weight.

7) Application: Security, search, and rescue on urban environment with stairs.

8) Publication – Normal(Journal & conference)

A. H. Adiwahono, B. Saputra, T. W. Chang, Z. X. Yong, “Autonomous Stair Identification, Climbing, and Descending for Tracked Robots”, 13th International Conference on Control, Automation, Robotics and Vision (ICARCV), 2014.